Gebruik emissiefactoren

Met een emissiefactor kan in combinatie met activiteitsgegevens, zoals productie of brandstofverbruik, relatief eenvoudig de consistentie van een gerapporteerde emissies (of het terecht niet rapporteren) worden gecontroleerd. Hierbij is het wel essentieel dat een representatieve emissiefactor wordt gebruikt.

De algemene formule voor het bepalen van vrachten met behulp van emissiefactoren is:

E = A x EF x (1-ER/100)

waarbij:

- E = emissie;

- A = activiteitsgegevens (gebruikte brandstof, productieomvang enz.);

- EF = emissiefactor, en

- ER =overall emissie reductie efficiëntie, %

Op basis van kennis en typering van een proces kunnen emissiefactoren worden geselecteerd, waarbij aandachtspunten zijn:

- Actualiteit; is de emissiefactor niet gedateerd?

- Representativiteit type proces; wijkt het proces niet teveel af van de processen waarvoor de emissiefactor is bepaald.

- Gebruik reducerende maatregelen; is al dan niet rekening gehouden met de toepassing van emissiereducerende maatregelen in het bepalen van de emissiefactor.

In bijlage 1 wordt een uitgebreid overzicht gegeven van informatiebronnen waar emissiefactoren gevonden kunnen worden. De volgende bronnen zijn er uitgelicht, voor deze is in bijlage 2 een koppeling naar E-PRTR activiteiten gegeven:

- BREF's referentiedocumenten best beschikbare techniek per IPPC activiteit.

- EMEP/CORINAIR Emission Inventory Guidebook – 2007, uitgebreide bronnen waarin voor een groot aantal processen emissiefactoren beschikbaar zijn. (CORINAIR = Core Inventory of Air Emissions)

- US EPA AP-42, overzicht van emissiefactoren verzameld door de US Environmental Protection Agency.

Bijlage 1	Overzicht informatiebronnen emissiefactoren

## Bijlage 2	Reference list for emission factors for E-PRTR activities

Reference list for CORINAIR, US EPA AP-42 and BAT Documents.

| E-PRTR activities (Annex I of the Regulation) | Corresponding CORINAIR (2004, III-rd edition) activities, SNAP CODE | EPA AP 42 | BREFs |
| --- | --- | --- | --- |
| 1. Energy sector |  |  |  |
| a) Mineral oil and gas refineries | 010306 Process furnaces without contact 0401 Processes in petroleum industries 040101 Petroleum products processing 040102 Fluid catalytic cracking – CO boiler 040103 Sulphur recovery plants 040104 Storage and handling of products in refinery 090203 Flaring in oil refinery 090206 Flaring in gas and oil extraction | Chapter 5. PETROLEUM INDUSTRY 5.1 Petroleum Refining 5.3 Natural Gas Processing | Refineries BREF (02-2003) |
| b) Installations for gasification and liquefaction | 010407 Other (coal gasification, liquefaction.) | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.11 Coal Conversion |  |
| c) Thermal power stations and other combustion installations | 0101 Public sector/power (Combustion in energy and transformation industries) 0102 District heating plants 0103 Petroleum refining plants 0104 Solid fuel transformation plants 0105 Coal mining, oil/gas extraction, pipeline compressors 0301 Combustion in boilers, gas turbines and stationary engines | Chapter 1. EXTERNAL COMBUSTION SOURCES 1.1 Bituminous And Sub-bituminous Coal Combustion 1.2 Anthracite Coal Combustion 1.3 Fuel Oil Combustion 1.4 Natural Gas Combustion 1.5 Liquefied Petroleum Gas Combustion 1.6 Wood Waste Combustion In Boilers 1.7 Lignite Combustion 1.8 Bagasse Combustion In Sugar Mills 1.9 Residential Fireplaces 1.10 Residential Wood Stoves 1.11 Waste Oil Combustion Chapter 3. STATIONARY INTERNAL COMBUSTION SOURCES 3.1 Stationary Gas Turbines For Electricity Generation 3.2 Heavy-duty Natural Gas-fired Pipeline Compressor Engines 3.3 Gasoline And Diesel Industrial Engines 3.4 Large Stationary Diesel And All Stationary Dual-fuel Engines | Large Combustion Plant BREF (07-2006) |
| d) Coke ovens | 010406 Coke oven furnaces (batteries) 040201 Coke oven (door leakage and extinction) | Chapter 12. METALLURGICAL INDUSTRY 12.2 Coke Production | Iron and Steel production BREF (12-2001) Ferrous Metal processing BREF (12-2001) |
| e) Coal rolling mills | No activity found. | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.10 Coal Cleaning | Management of Tailings and Waste-Rock in Mining Activities BREF (07-2004) |
| f) Installations for the manufacture of coal products and solid smokeless fuel | 040204 Solid smokeless fuel | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.10 Coal Cleaning | Management of Tailings and Waste-Rock in Mining Activities BREF (07-2004) |
| 2. Production and processing of metals |  |  |  |
| a) Metal ore (including sulphide ore) roasting or sintering installations | 030301 Sinter and pelletizing plants 030304 Primary lead production 030305 Primary zinc production 030306 Primary copper production | Chapter 12. METALLURGICAL INDUSTRY 12.3 Primary Copper Smelting 12.5 Iron And Steel Production 12.7 Zinc Smelting 12.6 Primary Lead Smelting 12.18 Lead-bearing Ore Crushing And Grinding | Management of Tailings and Waste-Rock in Mining Activities BREF (07-2004) |
| b) Installations for the production of pig iron or steel (primary or secondary melting) including continuous casting | 030203 Blast furnace cowpers 040202 Blast furnace charging 040203 Pig iron tapping 040205 Open hearth steel plant 040206 Basic oxygen furnace steel plant 040207 Electric furnace steel plant | Chapter 12. METALLURGICAL INDUSTRY 12.5 Iron And Steel Production 12.10 Gray Iron Foundries 12.13 Steel Foundries | Iron and Steel production BREF (12-2001) Ferrous Metal processing BREF (12-2001) |
| c) Installations for processing of ferrous metals: (i) Hot-rolling mills (ii) Smitheries with hammers (iii) Application of protective fused metal coats | 030302 Reheating furnaces steel and iron 040208 Rolling mills ( ingot) | Chapter 12. METALLURGICAL INDUSTRY 12.5 Iron And Steel Production 12.10 Gray Iron Foundries 12.13 Steel Foundries 12.4 Ferroalloy Production | Ferrous Metal processing BREF (12-2001) Smitheries and Foundries BREF (05-2005) |
| d) Ferrous metal foundries | 030303 Grey iron foundries | Chapter 12. METALLURGICAL INDUSTRY 12.10 Gray Iron Foundries | Ferrous Metal processing BREF (12-2001) Smitheries and Foundries BREF (05-2005) |
| e) Installations: (i) For the production of non-ferrous crude metals from ore, concentrates or secondary raw materials by metallurgical, chemical or electrolytic processes (ii) For smelting, including the alloying, of non-ferrous metals, including recovered products (refining, foundry, casting, etc.) | 030304 Primary lead production 030305 Primary zinc production 030306 Primary copper production 030307 Secondary lead production 030308 Secondary zinc production 030309 Secondary copper production 030310 Secondary aluminium production 030322 Alumina production 040301 Aluminium production (electrolysis) | Chapter 12. METALLURGICAL INDUSTRY 12.1 Primary Aluminium Production 12.3 Primary Copper Smelting 12.6 Primary Lead Smelting 12.7 Zinc Smelting 12.8 Secondary Aluminium Operations 12.9 Secondary Copper Smelting And Alloying 12.11 Secondary Lead Processing 12.12 Secondary Magnesium Smelting 12.14 Secondary Zinc Processing 12.18 Lead-bearing Ore Crushing And Grinding | Non-Ferrous Metal processes BREF (12-2001) |
| f) Installations for surface treatment of metals and plastic materials using an electrolytic or chemical process | - | Chapter 4. EVAPORATION LOSS SOURCES 4.2 Surface Coating 4.2.1 Non-industrial Surface Coating 4.2.2.2 Can Coating 4.2.2.1 General Industrial Surface Coating 4.2.2 Industrial Surface Coating 4.2.2.3 Magnet Wire Coating 4.2.2.4 Other Metal Coating 4.2.2.7 Polymeric Coating Of Supporting Substrates 4.2.2.8 Automobile And Light Duty Truck Surface Coating Operations 4.2.2.11 Large Appliance Surface Coating 4.2.2.10 Metal Coil Surface Coating 4.2.2.12 Metal Furniture Surface Coating 4.2.2.13 Magnetic Tape Manufacturing 4.2.2.14 Surface Coating Of Plastic Parts For Business Machines 4.2.2.5 Flat Wood Interior Panel Coating 4.2.2.6 Paper Coating 4.2.2.9 Pressure Sensitive Tapes And Labels Chapter 12. METALLURGICAL INDUSTRY 12.20 Electroplating | Surface treatment of metals BREF (08-2006) Surface treatments using solvents BREF (01-2007) |
| 3. Mineral industry |  |  |  |
| a) Underground mining and related operations | 050102 Underground mining 050103 Storage of solid fuel | - | Management of Tailings and Waste-Rock in Mining Activities BREF (07-2004) |
| b) Opencast mining and quarrying | 050101 Open cast mining 050103 Storage of solid fuel | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.9 Western Surface Coal Mining | Management of Tailings and Waste-Rock in Mining Activities BREF (07-2004) |
| c) Installations for the production of: (i) Cement clinker in rotary kilns (ii) Lime in rotary kilns (iii) Cement clinker or lime in other furnaces | 030311 Cement 030312 Lime | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.6 Portland Cement Manufacturing 11.17 Lime Manufacturing | Cement and Lime production BREF (12-2001) |
| d) Installations for the production of asbestos and the manufacture of asbestos-based products | - | - | - |
| e) Installations for the manufacture of glass, including glass fibre | 030314 Flat glass and 040613 Glass (decarbonising) 030315 Container glass 030316 Glass wool (except binding) 030317 Other glass (including special glass) | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.13 Glass Fibre Manufacturing 11.14 Frit Manufacturing 11.15 Glass Manufacturing | Glass manufacture BREF (12-2001) |
| f) Installations for melting mineral substances, including the production of mineral fibres | 030318 Mineral wool (except binding) | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.18 Mineral Wool Manufacturing | Glass manufacture BREF (12-2001) |
| g) Installations for the manufacture of ceramic products by firing, in particular roofing tiles, bricks, refractory bricks, tiles, stoneware or porcelain | 030319 Bricks and tiles 030320 Fine ceramics materials | Chapter 11. MINERAL PRODUCTS INDUSTRY 11.3 Bricks And Related Clay Products 11.5 Refractory Manufacturing 11.7 Ceramic Products Manufacturing | Ceramics BREF (12-2006) |
| 4. Chemical industry |  |  |  |
| a) Chemical installations for the production on an industrial scale of basic organic chemicals, such as: (i) Simple hydrocarbons (linear or cyclic, saturated or unsaturated, aliphatic or aromatic) (ii) Oxygen-containing hydrocarbons such as alcohols, aldehydes, ketones, carboxylic acids, esters, acetates, ethers, peroxides, epoxy resins (iii) Sulphurous hydrocarbons (iv) Nitrogenous hydrocarbons such as amines, amides, nitrous compounds, nitro compounds or nitrate compounds, nitriles, cyanates, isocyanates (v) Phosphorous-containing hydrocarbons (vi) Halogenic hydrocarbons (vii) Organometallic compounds (viii) Basic plastic materials (polymers, synthetic fibres and cellulose-based fibres) (ix) Synthetic rubbers (x) Dyes and pigments (xi) Surface-active agents and surfactants | 040501 Ethylene 040502 Propylene 040505 1,2 dichloroethane + vinylchloride (balanced process) 040506 Polyethylene low density 040507 Polyethylene high density 040508 Polyvinylchloride 040509 Polypropylene 040510 Styrene 040511 Polystyrene 040512 Styrene Butadiene 040513 Styrene- butadiene latex 040514 Styrene- Butadiene rubber (SBR) 040515 Acrylonitrile butadiene styrene (ABS) resins 040518 Ethylbenzene 040519 Phtalic anhydride 040520 Acrylonitrile 040521 Adipic acid | Chapter 6. ORGANIC CHEMICAL PROCESS INDUSTRY 6.1 Carbon Black 6.2 Adipic Acid 6.3 Explosives 6.4 Paint And Varnish 6.5 Phthalic Anhydride 6.6 Plastics 6.6.1 Polyvinyl Chloride 6.6.2 Poly(ethylene terephthalate) 6.6.3 Polystyrene 6.6.4 Polypropylene 6.7 Printing Ink 6.8 Soap And Detergents 6.9 Synthetic Fibres 6.10 Synthetic Rubber 6.11 Terephthalic Acid 6.12 Lead Alkyl 6.13 Pharmaceuticals Production 6.14 Maleic Anhydride 6.15 Methanol 6.16 Acetone And Phenol 6.17 Propylene 6.18 Benzene, Toluene And Xylenes 6.19 Butadiene 6.20 Cumene 6.21 Ethanol 6.22 Ethyl Benzene 6.23 Ethylene 6.24 Ethylene Dichloride And Vinyl Chloride 6.25 Ethylene Glycol 6.26 Ethylene Oxide 6.27 Formaldehyde 6.28 Glycerine 6.29 Isopropyl Alcohol Chapter 4. EVAPORATION LOSS SOURCES 4.4 Polyester Resin Plastic Products Fabrication | Large Volume Organic Chemicals BREF (02-2003) Organic fine chemicals BREF (08-2006) Polymers BREF (07-2006) Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| b) Chemical installations for the production on an industrial scale of basic inorganic chemicals, such as: (i) Gases, such as ammonia, chlorine or hydrogen chloride, fluoride, carbon oxides, sulphur compounds, nitrogen oxides, hydrogen, sulphur compounds, nitrogen oxides, hydrogen, sulphur dioxide, carbonyl chloride (ii) Acids, such as chromic acid, hydrofluoric acid, phosphoric acid, nitric acid, hydrochloric acid, sulphuric acid, oleum, Sulphurous acids (iii) Bases, such as ammonium chloride, potassium chlorate, potassium carbonate, sodium carbonate, perborate, silver nitrate (iv) Salts, such as ammonium chloride, potassium chlorate, potassium carbonate, sodium carbonate, perborate, silver nitrate (v) Non-metals, metal oxides or other inorganic compounds such as calcium carbide, silicon, silicon carbide | 040401 Production of sulphiric acid 040402 Production of nitric acid | Chapter 8. INORGANIC CHEMICAL INDUSTRY 8.1 Synthetic Ammonia 8.6 Hydrochloric Acid 8.7 Hydrofluoric Acid 8.8 Nitric Acid 8.9 Phosphoric Acid 8.10 Sulfuric Acid 8.11 Chlor-Alkali 8.12 Sodium Carbonate 8.13 Sulfur Recovery 8.14 Hydrogen Cyanide | Large Volume Inorganic Chemicals - Ammonia, Acids & Fertilisers BREF(12-2006) Large Volume Inorganic Chemicals - Solid & Others BREF (10-2006) Chlor-Alkali manufacture BREF (12-2001) Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| c) Chemical installations for the production on an industrial scale of phosphorous-, nitrogen- or potassium-based fertilisers (simple or compound fertilisers) | - | Chapter 8. INORGANIC CHEMICAL INDUSTRY 8.2 Urea 8.3 Ammonium Nitrate 8.4 Ammonium Sulfate 8.5 Phosphate Fertilizers 8.5.1 Normal Superphosphates 8.5.2 Triple Superphosphates 8.5.3 Ammonium Phosphate | Large Volume Inorganic Chemicals - Ammonia, Acids & Fertilisers BREF(12-2006) Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| d) Chemical installations for the production on an industrial scale of basic plant health products and of biocides | - | - | Organic fine chemicals BREF (08-2006) Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| e) Installations using a chemical or biological process for the production on an industrial scale of basic pharmaceutical products | - | Chapter 6. ORGANIC CHEMICAL PROCESS INDUSTRY 6.13 Pharmaceuticals Production | Organic fine chemicals BREF (08-2006) Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| f) Installations for the production on an industrial scale of explosives and pyrotechnic products | - | Chapter 6. ORGANIC CHEMICAL PROCESS INDUSTRY 6.3 Explosives | - |
| 5. Waste and wastewater management |  |  |  |
| a) Installations for the recovery or disposal of hazardous waste | - | Chapter 2. SOLID WASTE DISPOSAL 2.1 Refuse Combustion 2.3 Medical Waste Incineration | Waste Incineration BREF (08-2006) Waste Treatments BREF (08-2006) |
| b) Installations for the incineration of non-hazardous waste in the scope of Directive 2000/76/EC of the European Parliament and of the Council of 4 December 2000 on the incineration of waste | 090201 Incineration of domestic or municipal wastes 090202 Incineration of industrial waste (except flaring) 090205 Incineration of sludges form waste water treatment | Chapter 2. SOLID WASTE DISPOSAL 2.1 Refuse Combustion 2.2 Sewage Sludge Incineration 2.3 Medical Waste Incineration 2.5 Open Burning 2.6 Automobile Body Incineration | Waste Incineration BREF (08-2006) Waste Treatments BREF (08-2006) |
| c) Installations for the disposal of non-hazardous waste | - (091005 Compost production) | Chapter 2. SOLID WASTE DISPOSAL 2.1 Refuse Combustion 2.4 Landfills | Waste Incineration BREF (08-2006) Waste Treatments BREF (08-2006) |
| d) Landfills (excluding landfills of inert waste and landfills, which were definitely closed before 16.7.2001 or for which the after-care phase required by the competent authorities according to Article 13 of Council Directive 1999/31/EC of 26 April 1999 on the landfill of waste has expired) | 090401 Managed waste disposal on land 090402 Unmanaged waste disposal on land 090403 Other | Chapter 2. SOLID WASTE DISPOSAL 2.4 Landfills | Waste Treatments BREF (08-2006) |
| e) Installations for the disposal or recycling of animal carcasses and animal waste | - | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.5 Animal And Meat Products Preparation 9.5.1 Meat Packing Plants 9.5.2 Meat Smokehouses 9.5.3 Meat Rendering Plants 9.5.4 Manure Processing 9.5.5 Poultry Slaughtering | Slaughterhouses and Animal By-products BREF (05-2005) |
| f) Urban waste-water treatment plants | 091002 Waste water treatment in residential and commercial sectors 091003 Sludge spreading | Chapter 4. EVAPORATION LOSS SOURCES 4.3 Waste Water Collection, Treatment And Storage |  |
| g) Independently operated industrial waste-water treatment plants which serve one or more activities of this annex | 091001 Waste water treatment in industry 091003 Sludge spreading | Chapter 4. EVAPORATION LOSS SOURCES 4.3 Waste Water Collection, Treatment And Storage | Common waste water and waste gas treatment and management systems in the chemical sector BREF (02-2003) |
| 6. Paper and wood production and processing |  |  |  |
| a) Industrial plants for the production of pulp from timber or similar fibrous materials | 040602 Paper pulp (Кraft process) 040603 Paper pulp (acid sulphite process) 040604 Paper pulp (neutral sulphite semi-chemical process) | Chapter 10. WOOD PRODUCTS INDUSTRY 10.2 Chemical Wood Pulping 10.3 Pulp Bleaching 10.4 Papermaking | Pulp and Paper manufacture BREF (12-2001) |
| b) Industrial plants for the production of paper and board and other primary wood products (such as chipboard, fibreboard and plywood) | 030321 Paper-mill industry (drying processes) | Chapter 10. WOOD PRODUCTS INDUSTRY 10.4 Papermaking 10.5 Plywood 10.6 Reconstituted Wood Products 10.6.1 Waferboard And Oriented Strand Board 10.6.2 Particleboard 10.6.3 Medium Density Fiberboard | Pulp and Paper manufacture BREF (12-2001) |
| c) Industrial plants for the preservation of wood and wood products with chemicals | 060406 Preservation of wood 060107 Painting of wood | Chapter 10. WOOD PRODUCTS INDUSTRY 10.8 Wood Preserving | - |
| 7. Intensive livestock production and aquaculture |  |  |  |
| a) Installations for the intensive rearing of poultry or pigs (i) With 40 000 places for poultry (ii) With 2 000 places for the production pigs (over 30 kg) (iii) With 750 places for sows | 100507 Laying hens 100508 Broilers 100509 Other poultry (ducks, geese, etc.) 100503 Fattening pigs 100504 Sows 100907 Laying hens 100908 Broilers 100909 Other poultry (ducks, geese, etc.) 100903 Fattening pigs 100904 Sows | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.4 Livestock And Poultry Feed Operations 9.4.1 Cattle Feedlots 9.4.2 Swine Feedlots 9.4.3 Poultry Houses 9.4.4 Dairy Farms | Intensive Livestock Farming BREF (07-2003) |
| b) Intensive aquaculture | - |  | - |
| 8. Animal and vegetable products from the food and beverage sector |  |  |  |
| a) Slaughterhouses | - | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.5.5 Poultry Slaughtering | Slaughterhouses and Animal By-products BREF (05-2005) |
| b) Treatment and processing intended for the production of food and beverage products from: (i) Animal raw materials (other than milk) (ii) Vegetable raw materials | - | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.5 Animal And Meat Products Preparation 9.5.1 Meat Packing Plants 9.5.2 Meat Smokehouses 9.5.3 Meat Rendering Plants 9.5.4 Manure Processing 9.5.5 Poultry Slaughtering 9.8 Preserved Fruits And Vegetables 9.8.1 Canned Fruits And Vegetables 9.8.2 Dehydrated Fruits And Vegetables 9.8.3 Pickles, Sauces And Salad Dressings 9.9 Grain Processing 9.9.1 Grain Elevators And Processes 9.9.2 Cereal Breakfast Food 9.9.3 Pet Food 9.9.5 Pasta Manufacturing 9.9.6 Bread Baking 9.9.7 Corn Wet Milling 9.10 Confectionery Products 9.10.1 Sugar Processing 9.10.1.1 Cane Sugar Processing 9.10.1.2 Beet Sugar Processing 9.10.2 Salted And Roasted Nuts And Seeds 9.10.2.1 Almond Processing 9.10.2.2 Peanut Processing 9.11 Fats And Oils 9.11.1 Vegetable Oil Processing 9.12 Beverages 9.12.1 Malt Beverages 9.12.2 Wines And Brandy 9.12.3 Distilled Spirits 9.13 Miscellaneous Food And Kindred Products 9.13.1 Fish Processing 9.13.2 Coffee Roasting 9.13.3 Snack Chip Deep Fat Frying 9.13.4 Yeast Production 9.14 Tobacco Products | Food, Drink and Milk processes BREF (08-2006) |
| c) Treatment and processing of milk | - | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.6 Dairy Products 9.6.1 Natural And Processed Cheese | Food, Drink and Milk processes BREF (08-2006) |
| 9. Other activities |  |  |  |
| a) Plants for the pre-treatment (operators such as washing, bleaching, mercerisation) or dyeing of fibres or textiles | - | Chapter 4. EVAPORATION LOSS SOURCES 4.1 Dry Cleaning 4.11 Textile Fabric Printing | Textile processing BREF (07-2003) |
| b) Plants for the tanning of hides and skins | - | Chapter 9. FOOD AND AGRICULTURAL INDUSTRIES 9.15 Leather Tanning | Tanning of hides and skins BREF (02-2003) |
| c) Installations for the surface treatment of substances, objects or products using organic solvents, in particular for dressing, printing, coating, degreasing, waterproofing, sizing, painting, cleaning or impregnating | 060201 Metal degreasing 060202 Dry cleaning 060406 Preservation of wood 060101 Painting in automobile manufacturing 060102 Painting in car repairing 060103 Painting in construction and buildings (except 060107) 060104 Painting in domestic use (except 060107) 060105 Painting in coil coating 060106 Painting in boat building 060107 Painting of wood 060108 Other industrial paint application 060109 Other non industrial paint application | Chapter 4. EVAPORATION LOSS SOURCES 4.2 Surface Coating 4.2.1 Non-industrial Surface Coating 4.2.2 Industrial Surface Coating 4.2.2.1 General Industrial Surface Coating 4.2.2.2 Can Coating 4.2.2.3 Magnet Wire Coating 4.2.2.4 Other Metal Coating 4.2.2.5 Flat Wood Interior Panel Coating 4.2.2.6 Paper Coating 4.2.2.7 Polymeric Coating Of Supporting Substrates 4.2.2.8 Automobile And Light Duty Truck Surface Coating Operations 4.2.2.9 Pressure Sensitive Tapes And Labels 4.2.2.10 Metal Coil Surface Coating 4.2.2.11 Large Appliance Surface Coating 4.2.2.12 Metal Furniture Surface Coating 4.2.2.13 Magnetic Tape Manufacturing 4.2.2.14 Surface Coating Of Plastic Parts For Business Machines 4.6 Solvent Degreasing 4.7 Waste Solvent Reclamation 4.8 Tank And Drum Cleaning 4.10 Commercial/Consumer Solvent Use | Surface treatment of metals BREF (08-2006) Surface treatments using solvents BREF (01-2007) |
| d) Installations for the production of carbon (hard-burnt coal) or electro-graphite by means of incineration or graphitization | - | Chapter 10. WOOD PRODUCTS INDUSTRY 10.7 Charcoal | - |
| e) Installations for the building of, and painting or removal of paint from ships | 060106 Painting in boat building | - | - |